TimeLapse Pro

Edge Agent — Installations-Runbook v3.0

| Felt | Værdi |
| --- | --- |
| Hardware | Orange Pi 4 Pro (RK3588S) + 128GB NVMe + HW-383A relay |
| OS | Ubuntu 24.04 LTS |
| GPIO | Direkte sysfs — GPIO 356 (kamera, pin 7), GPIO 361 (modem, pin 11). OPi.GPIO INKOMPATIBEL. |
| Kamera | Canon EOS 1300D — USB PTP via gphoto2. gvfs-gphoto2-volume-monitor skal deaktiveres. |
| Headend | Raspberry Pi 5 — 192.168.86.132:8000 |
| Dato | 31. marts 2026 |

# 1. Forudsætninger

## 1.1 Hardware tjekliste

- Orange Pi 4 Pro tændt og tilgængelig via SSH (orangepi@192.168.86.134)

- NVMe M.2 SSD monteret — booter fra NVMe (tjek: lsblk — nvme0n1 som root)

- Canon EOS 1300D tilsluttet USB-kabel

- HW-383A relay modul: VCC=5V (pin 2), GND (pin 6), IN1=pin 7, IN2=pin 11

- Netværksforbindelse — ethernet anbefales til installation

## 1.2 Kritiske GPIO-oplysninger (RK3588S specifikt)

OPi.GPIO BCM/BOARD er INKOMPATIBEL med Orange Pi 4 Pro (RK3588S). Brug altid direkte sysfs.

| Funktion | Physical pin | sysfs GPIO | HW-383A kanal |
| --- | --- | --- | --- |
| Kamera | Pin 7 | GPIO 356 | IN1 — Relay 1 |
| Modem | Pin 11 | GPIO 361 | IN2 — Relay 2 |

HW-383A er aktiv-lav: GPIO LOW (0V) = relay ON = strøm til enhed.

# 2. Installation

## 2.1 Klargør storage og mapper

sudo mkdir -p /data/captures /data/sftp

sudo chown -R orangepi:orangepi /data

# tmpfiles — /run/timelapse overlever reboot

echo 'd /run/timelapse 0755 root root -' | sudo tee /etc/tmpfiles.d/timelapse.conf

## 2.2 Installer Python-afhængigheder

sudo apt install -y python3-venv python3-pip

python3 -m venv /opt/timelapse/venv

/opt/timelapse/venv/bin/pip install fastapi uvicorn paramiko pyyaml requests opencv-python-headless

## 2.3 KRITISK — Deaktiver gvfs-gphoto2-volume-monitor

gvfs-gphoto2-volume-monitor blokerer gphoto2's USB-adgang og forhindrer kamera-diagnostik. MÅ deaktiveres.

# Som orangepi-bruger (ikke sudo):

DBUS_SESSION_BUS_ADDRESS='unix:path=/run/user/1000/bus' systemctl --user stop gvfs-gphoto2-volume-monitor

mkdir -p ~/.config/systemd/user

systemctl --user mask gvfs-gphoto2-volume-monitor

# Verificer:

ps aux | grep gphoto2 | grep -v grep  # Skal ikke vise gvfs-gphoto2-volume-monitor

## 2.4 Installer NTP/chrony

sudo apt install -y chrony

sudo systemctl enable --now chrony

chronyc tracking  # Verificer offset < 1 sekund

## 2.5 Konfigurer systemd service

sudo tee /etc/systemd/system/timelapse-edge.service << 'EOF'

[Unit]

Description=TimeLapse Pro Edge Agent

After=network.target

[Service]

User=root

WorkingDirectory=/opt/timelapse/edge

ExecStartPre=/bin/sh -c "echo 356 > /sys/class/gpio/unexport 2>/dev/null || true; echo 361 > /sys/class/gpio/unexport 2>/dev/null || true"

ExecStart=/opt/timelapse/venv/bin/python /opt/timelapse/edge/agent.py

Restart=always

RestartSec=10

[Install]

WantedBy=multi-user.target

EOF

sudo systemctl daemon-reload

sudo systemctl enable timelapse-edge

## 2.6 Nightly reboot

echo '0 3 * * * root /sbin/reboot' | sudo tee /etc/cron.d/timelapse-reboot

# 3. Konfiguration

## 3.1 config.yaml — Komplet eksempel

device:

device_id: TL-C87FF9587CA0  # Auto-genereres fra MAC

headend_url: http://192.168.86.132:8000/api

location_name: KundeA — SiteB — Kamera 1

customer_name: KundeA

site_name: SiteB

camera_name: Kamera 1

schedule:

timezone: Europe/Copenhagen

capture_mode: interval    # eller 'fixed'

interval_minutes: 10

active_hours: ['06:00', '21:00']

# capture_times: ['08:00', '12:00', '16:00']  # Fixed mode

camera:

relay_gpio_pin: 356         # sysfs GPIO

relay_on_seconds_before: 10

relay_off_seconds_after: 5

relay_simulate: false

gphoto2_port: 'usb:'

delete_after_download: true

initial_commands:           # Køres ved hvert camera connect

- 'capturetarget=1'       # Memory card (KRITISK — Canon nulstiller)

- 'iso=0'                 # Auto ISO

- 'colorspace=0'          # sRGB

- 'meteringmode=0'        # Evaluative

modem:

modem_relay_gpio_pin: 361

modem_cycle_after_failures: 3

sftp:

host: 192.168.86.132

port: 22

username: sftp_test

password: timelapse123    # Udskiftes med SSH-nøgle i Fase 2

remote_base: /incoming

## 3.2 Sæt enhedsidentitet på headend (obligatorisk FØR første capture)

Enhedsidentiteten indgår i filnavnet — sæt den inden første capture!

curl -X PUT http://192.168.86.132:8000/api/admin/devices/TL-C87FF9587CA0/info \

-H 'Content-Type: application/json' \

-d '{"customer_name": "KundeA", "site_name": "SiteB", "camera_name": "Kamera1",

"installed_date": "2026-03-31", "installed_time": "12-00"}'

## 3.3 Sæt initial_commands via headend

curl -X PUT http://192.168.86.132:8000/api/admin/devices/TL-C87FF9587CA0/config \

-H 'Content-Type: application/json' \

-d '{"camera": {"initial_commands": ["capturetarget=1","iso=0","colorspace=0","meteringmode=0"]}}'

# 4. NTP Tidssynkronisering

## 4.1 Verificer chrony

timedatectl status | grep -E 'NTP|synchronized'

chronyc tracking   # System time viser offset

chronyc sources    # Viser NTP-servere

## 4.2 Acceptabel offset

| Krav | Metode | Typisk offset |
| --- | --- | --- |
| < 1 sekund (krav) | chrony over internet | ~2-500ms — opnås altid |
| < 100ms (anbefalet) | chrony med LAN NTP-server | ~2-50ms |
| Nuværende system | chrony, Stratum 2 | ~0.2ms (0.0002s) — fremragende |

# 5. Test og Verificering

## 5.1 Test GPIO relay

sudo python3 -c "

from pathlib import Path; import time

g = Path('/sys/class/gpio')

try: (g/'export').write_text('356')

except: pass

(g/'gpio356/direction').write_text('out')

(g/'gpio356/value').write_text('0'); print('Relay ON')

time.sleep(3)

(g/'gpio356/value').write_text('1'); print('Relay OFF')

"

## 5.2 Test kamera-diagnostik

sudo /opt/timelapse/venv/bin/python -c "

import sys; sys.path.insert(0, '/opt/timelapse/edge')

from diagnostics.camera_diagnostics import collect_camera_diagnostics

r = collect_camera_diagnostics('Canon EOS 1300D', {})

print('battery:', r['camera_status'].get('battery_pct'))

print('shutter:', r['camera_status'].get('shutter_count'))

print('drift:', r['camera_config_drift'])

"

Kameraet skal være tændt (relay ON) og gvfs-gphoto2-volume-monitor SKAL være deaktiveret.

## 5.3 Test agent — første capture

sudo systemctl start timelapse-edge

journalctl -u timelapse-edge -f | grep -E 'Captured|relay ON|Heartbeat|drift|shutter'

Forventet sekvens:

- Camera relay ON (GPIO 356)

- Camera detected: Canon EOS 1300D

- Applying 4 camera commands → Camera commands applied OK

- Captured: KundeA_SiteB_Kamera1_YYYYMMDD_HHMMSS.jpg

- Camera diagnostics collected: battery=100 shutter=XXXXX drift=0

- Heartbeat sent OK

- Camera relay OFF (GPIO 356)

## 5.4 Verificer filnavn

Filnavnet SKAL indeholde enhedsidentitet:

ls /data/captures/ | tail -3

Forventet: KundeA_SiteB_Kamera1_20260331_120000.jpg

Fejl (MAC-baseret): TL-C87FF9587CA0_20260331_120000.jpg → Tjek customer_name i config.yaml

# 6. Fejlfinding

## 6.1 Almindelige problemer

| Problem | Årsag | Løsning |
| --- | --- | --- |
| OSError [Errno 16] Device busy | GPIO allerede exporteret | ExecStartPre i service unexporter GPIO 356+361 ved opstart |
| Permission denied /sys/class/gpio | Kører ikke som root | Service SKAL køre som User=root |
| camera_diagnostics hænger | gvfs-gphoto2-volume-monitor kører | systemctl --user mask gvfs-gphoto2-volume-monitor (se afsnit 2.3) |
| Camera not found | gvfs eller USB-konflikt | Verificer gvfs deaktiveret: ps aux ¦ grep gvfs-gphoto2 |
| Filnavn er MAC-baseret | __pycache__ eller manglende navne | find /opt/timelapse/edge -name '__pycache__' -exec rm -rf {} + |
| Config springer ikke over ved genstart | /run/timelapse mangler | systemd-tmpfiles --create /etc/tmpfiles.d/timelapse.conf |
| NTP ikke synkroniseret | chrony ikke installeret | sudo apt install chrony && sudo systemctl enable --now chrony |
| Billede tages 10-15 sek. efter X:00 | Fixed mode ikke konfigureret | Sæt capture_mode: fixed + capture_times i headend config |
| capturetarget = Internal RAM fejl | initial_commands mangler | PUT /api/admin/devices/{id}/config med capturetarget=1 i initial_commands |

## 6.2 Diagnostikkommandoer

journalctl -u timelapse-edge -f                       # Live log

gpio readall                                           # GPIO-mapping (wPi 2=GPIO 356, wPi 5=361)

chronyc tracking                                       # NTP offset

ps aux | grep gvfs-gphoto2                            # Verificer deaktiveret

sqlite3 /data/timelapse_edge.db 'SELECT COUNT(*) FROM captures;'

curl -s http://192.168.86.132:8000/health             # Headend liveness

curl -s http://192.168.86.132:8000/api/admin/stats | python3 -m json.tool

## 6.3 Kodeændringer — HUSK altid

Efter enhver kodeændring på Orange Pi:

find /opt/timelapse/edge -name '*.pyc' -delete

find /opt/timelapse/edge -name '__pycache__' -type d -exec rm -rf {} + 2>/dev/null || true

sudo systemctl restart timelapse-edge