TimeLapse Pro

Distributed Timelapse Capture System

System Architecture & Configuration Guide

Headend Server  ·  Camera Location (Edge)

| Version | 1.0 — Initial Release |
| --- | --- |
| Architecture | SABSA-aligned |
| Platform | Ubuntu 24.04 LTS / Orange Pi 4 Pro / Raspberry Pi 5 |
| Language | Python 3.11+ |
| Date | 28 March 2026 |

# 1.  System Overview

## 1.1  Architecture Summary

TimeLapse Pro is a multi-tenant, distributed timelapse capture system. Remote edge nodes (Orange Pi 4 Pro) autonomously capture images from Canon DSLR cameras via gPhoto2, assess image quality, store them in a local circular buffer, and synchronise with a central headend over SFTP and a REST API.

### System Architecture (SABSA Layers)

| SABSA Layer | Concern | Implementation |
| --- | --- | --- |
| Contextual | Why | Reliable, unattended, multi-customer timelapse capture from hard-to-reach locations |
| Conceptual | What | Availability · Integrity · Confidentiality · Resilience · Auditability |
| Logical | How | Zero-trust edge↔headend; API auth tokens; encrypted SFTP; local autonomy on disconnect |
| Physical | With What | Orange Pi 4 Pro, Canon EOS 1300, Ubuntu, Python, SQLite (edge), PostgreSQL (headend) |
| Component | Build | gphoto2, systemd, FastAPI, Nginx, SFTP, SQLite/PostgreSQL, React/Jinja2 web UI |
| Operational | Run | Watchdog daemons, health heartbeats, reverse SSH tunnels, circular buffer, relay control |

## 1.2  Component Map

The system comprises two physical tiers:

- Headend (Central Control): Ubuntu server (Mac in production / Raspberry Pi 5 in test). Runs the REST API, web interface, PostgreSQL database, SFTP server, and email dispatch service.

- Edge Node (Camera Location): Orange Pi 4 Pro running Ubuntu, with 128 GB M.2 SSD, connected to a Canon EOS 1300 via USB and internet via WiFi / Ethernet / 5G USB modem.

Key Design Decisions Edge nodes operate fully standalone when the headend is unreachable (local config cache, local SQLite DB, local circular buffer). Edge devices are untrusted by default — bootstrap authenticates via MAC-derived device ID and a signed API token. Every picture is logged with a SHA-256 hash, quality score, camera state, voltage and temperature. Power relay control ensures the camera is only energised during capture windows, maximising battery life. Reverse SSH tunnels allow the headend to reach edge nodes that are behind NAT/CGNAT.

## 1.3  Network & Trust Boundaries

| Connection | Details |
| --- | --- |
| Edge → Headend SFTP | TLS/SFTP on port 22. Each location has a dedicated OS account with chroot jail. Camera subfolders created by edge. |
| Edge → Headend API | HTTPS on port 443. JWT bearer token issued at bootstrap. Tokens rotated on schedule. |
| Headend → Edge SSH | Reverse tunnel initiated by edge on config request. Headend connects back on a dynamic high port. |
| Internet connectivity | Edge supports Ethernet, WiFi (wpa_supplicant) and USB 5G modem (NetworkManager / ModemManager) with automatic failover. |
| Camera ↔ Edge | USB (libgphoto2). Camera powered via relay GPIO. No network exposure. |

# 2.  Headend Configuration

## 2.1  Prerequisites & OS Setup

Tested on Ubuntu 22.04 LTS and Ubuntu 24.04 LTS. All commands run as root or via sudo.

### 2.1.1  System Update

sudo apt update && sudo apt upgrade -y

sudo apt install -y python3.11 python3.11-venv python3-pip postgresql nginx

sudo apt install -y openssh-server git curl ufw fail2ban

### 2.1.2  Firewall Rules

sudo ufw allow 22/tcp # SSH

sudo ufw allow 80/tcp # HTTP (redirects to HTTPS)

sudo ufw allow 443/tcp # HTTPS API + Web UI

sudo ufw allow 2222/tcp # Dedicated SFTP port (optional)

sudo ufw enable

## 2.2  PostgreSQL Database

### 2.2.1  Create Database & User

sudo -u postgres psql

CREATE USER timelapse WITH PASSWORD 'CHANGE_ME_STRONG_PASSWORD';

CREATE DATABASE timelapse_db OWNER timelapse;

GRANT ALL PRIVILEGES ON DATABASE timelapse_db TO timelapse;

\q

### 2.2.2  Schema Overview

The following tables are created automatically by the API service on first run (SQLAlchemy migrations):

| Table | Purpose |
| --- | --- |
| tenants | Multi-tenant isolation. Each customer is a tenant with their own API key prefix. |
| locations | Physical locations (sites). Belong to a tenant. Has SFTP account details. |
| devices | Edge nodes identified by MAC address. Has device_id, current config_url, last_seen. |
| cameras | Cameras at a location. Has model, serial, current config (JSON), relay GPIO. |
| captures | One row per captured image. SHA-256 hash, quality score, file path, timestamps. |
| diagnostics | Time-series: voltage, temperature, CPU load, disk usage, connectivity state. |
| api_tokens | JWT token records. Issued at bootstrap, rotatable, linked to device. |
| config_versions | Configuration history for audit trail. Every config push is versioned. |

## 2.3  SFTP Server (Per-Location Accounts)

### 2.3.1  SFTP Chroot Setup

Each customer location receives a dedicated OS user account. Files land in a chroot jail under /data/sftp/<tenant>/<location>/.

# Create SFTP group

sudo groupadd sftpusers

# Per-location: run the provisioning script (see Section 2.6)

sudo /opt/timelapse/headend/scripts/create_sftp_user.sh \

--tenant acme --location site01 --pubkey /tmp/edge_id_rsa.pub

### 2.3.2  /etc/ssh/sshd_config additions

# Add at end of /etc/ssh/sshd_config

Match Group sftpusers

ChrootDirectory /data/sftp/%u

ForceCommand internal-sftp

AllowTcpForwarding no

X11Forwarding no

sudo systemctl restart sshd

# Directory structure created by provisioning script:

# /data/sftp/<tenant>_<location>/ (owned root:root, 755 — chroot requirement)

# /data/sftp/<tenant>_<location>/incoming/ (owned sftpuser, 755)

# /data/sftp/<tenant>_<location>/incoming/<camera_id>/ (created by edge on first connect)

## 2.4  REST API Service

### 2.4.1  Installation

sudo mkdir -p /opt/timelapse/headend

sudo python3.11 -m venv /opt/timelapse/venv

source /opt/timelapse/venv/bin/activate

pip install fastapi uvicorn sqlalchemy psycopg2-binary python-jose

pip install passlib bcrypt pydantic-settings python-multipart aiofiles

pip install paramiko Pillow numpy scipy

### 2.4.2  Environment Configuration  /opt/timelapse/headend/.env

DATABASE_URL=postgresql://timelapse:CHANGE_ME@localhost/timelapse_db

SECRET_KEY=GENERATE_WITH__openssl rand -hex 32

SFTP_ROOT=/data/sftp

PICTURE_ARCHIVE=/data/pictures

SMTP_HOST=smtp.example.com

SMTP_PORT=587

SMTP_USER=timelapse@example.com

SMTP_PASS=CHANGE_ME

SMTP_FROM=timelapse@example.com

REVERSE_SSH_BASE_PORT=40000

ALLOWED_ORIGINS=https://timelapse.example.com

### 2.4.3  Systemd Service  /etc/systemd/system/timelapse-api.service

[Unit]

Description=TimeLapse Pro Headend API

After=network.target postgresql.service

[Service]

User=timelapse

WorkingDirectory=/opt/timelapse/headend

EnvironmentFile=/opt/timelapse/headend/.env

ExecStart=/opt/timelapse/venv/bin/uvicorn app.main:app \

--host 127.0.0.1 --port 8000 --workers 4

Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

sudo systemctl daemon-reload

sudo systemctl enable --now timelapse-api

## 2.5  Nginx Reverse Proxy

### 2.5.1  /etc/nginx/sites-available/timelapse

server {

listen 80;

server_name timelapse.example.com;

return 301 https://$host$request_uri;

}

server {

listen 443 ssl http2;

server_name timelapse.example.com;

ssl_certificate /etc/letsencrypt/live/timelapse.example.com/fullchain.pem;

ssl_certificate_key /etc/letsencrypt/live/timelapse.example.com/privkey.pem;

ssl_protocols TLSv1.2 TLSv1.3;

location /api/ {

proxy_pass http://127.0.0.1:8000/;

proxy_set_header Host $host;

proxy_set_header X-Real-IP $remote_addr;

}

location / {

root /opt/timelapse/headend/static;

try_files $uri $uri/ /index.html;

}

}

sudo ln -s /etc/nginx/sites-available/timelapse /etc/nginx/sites-enabled/

sudo certbot --nginx -d timelapse.example.com # Let's Encrypt TLS

sudo systemctl reload nginx

## 2.6  API Endpoints — Key Routes

| Endpoint | Description |
| --- | --- |
| POST /bootstrap | Edge presents MAC address + device_id. Returns JWT token and config URL. Creates device record if new. |
| GET /config/{device_id} | Returns the current configuration JSON for the device. |
| POST /heartbeat/{device_id} | Edge posts diagnostics (voltage, temp, disk, connectivity). Updates last_seen. |
| POST /reverse-ssh/ready | Edge notifies headend that reverse SSH tunnel is up, with dynamic port number. |
| POST /captures/{device_id} | Metadata record for a captured image (hash, quality, timestamp, camera state). |
| GET /admin/devices | Web UI: list all devices with last-seen, status, diagnostics. |
| GET /admin/captures | Web UI: browse captures with filtering by tenant / location / camera / date. |
| POST /admin/config/{device_id} | Push new configuration to a device. Stored versioned. |
| POST /email/report | Trigger email report for a location (pictures, quality stats, diagnostics). |

## 2.7  Web Interface

The web interface is a Python Jinja2 application (or React SPA) served by Nginx. It is multi-tenant: each tenant logs in and sees only their locations, devices, and captures.

- Dashboard: all devices, last heartbeat, online/offline status, quality trend.

- Location view: list of cameras, latest captures as thumbnails, quality scores.

- Camera detail: capture timeline, quality graph, diagnostic charts (voltage, temperature).

- Configuration editor: edit and push YAML/JSON configuration with version history.

- Reports: trigger or schedule email reports per location.

- Admin: tenant management, SFTP account provisioning, API token management.

⚠ Security Reminder Change all default passwords and the SECRET_KEY before going to production. Enable fail2ban for SSH and the API (rate-limit /bootstrap endpoint). Rotate JWT tokens on a schedule — configure token_ttl_hours in the API settings. Restrict database access to localhost only (PostgreSQL pg_hba.conf).

# 3.  Edge Node Configuration  (Camera Location)

## 3.1  Hardware Setup

| Component | Specification |
| --- | --- |
| SBC | Orange Pi 4 Pro (8 GB RAM recommended) |
| Storage | 128 GB M.2 NVMe SSD (mounted at /data) |
| Camera | Canon EOS 1300D connected via USB |
| Relay | GPIO-controlled relay (BCM pin configurable) for camera power |
| Connectivity | Ethernet (primary) / WiFi (wpa_supplicant) / USB 5G modem (ModemManager) |
| Power | 230V mains OR 12V battery + solar panel via DC-DC converter |
| Enclosure | Weatherproof housing on mast. Camera window: polycarbonate or glass. |

⚠ Physical Security Use stainless steel fasteners — the mast environment is corrosive. Apply conformal coating to the relay PCB and any exposed connectors. Run the USB cable through a drip loop before entering the housing. Label the housing with the device_id and emergency contact number.

## 3.2  OS Installation

### 3.2.1  Ubuntu on Orange Pi 4 Pro

# Flash Ubuntu Server 22.04 image for Orange Pi 4 Pro

# https://github.com/orangepi-xunlong/orangepi-build

# Or use official image from orangepi.org

# First boot — expand filesystem, set hostname

sudo hostnamectl set-hostname timelapse-<device_id>

sudo apt update && sudo apt upgrade -y

sudo apt install -y python3.11 python3.11-venv python3-pip gphoto2 libgphoto2-dev

sudo apt install -y sqlite3 curl openssh-client network-manager modemmanager

sudo apt install -y watchdog i2c-tools python3-rpi.gpio python3-smbus

### 3.2.2  Mount the M.2 SSD

# Identify the SSD

lsblk

# Partition and format

sudo parted /dev/nvme0n1 mklabel gpt

sudo parted /dev/nvme0n1 mkpart primary ext4 0% 100%

sudo mkfs.ext4 /dev/nvme0n1p1

# Get UUID and add to /etc/fstab

sudo blkid /dev/nvme0n1p1

echo 'UUID=<uuid> /data ext4 defaults,noatime 0 2' ¦ sudo tee -a /etc/fstab

sudo mkdir -p /data && sudo mount -a

### 3.2.3  GPIO Relay (Camera Power Control)

# Install GPIO library for Orange Pi

pip install OPi.GPIO # Orange Pi GPIO (drop-in RPi.GPIO compatible)

# Relay wiring: GPIO pin (BCM) → relay IN

# Relay COM → camera power input

# Relay NO (normally open) → camera power source

# Default relay GPIO pin: 18 (configurable in config file)

## 3.3  Software Installation

### 3.3.1  Install TimeLapse Edge Agent

sudo mkdir -p /opt/timelapse/edge

sudo python3.11 -m venv /opt/timelapse/venv

source /opt/timelapse/venv/bin/activate

pip install gphoto2 requests paramiko Pillow numpy scipy

pip install pyyaml schedule sqlite3 # sqlite3 is stdlib, pyyaml for config

# Clone or copy edge agent code

sudo git clone https://github.com/yourorg/timelapse-edge /opt/timelapse/edge

### 3.3.2  Bootstrap File  /opt/timelapse/edge/bootstrap.yaml

This file is written during device provisioning and identifies the device to the headend. It is never overwritten by the configuration sync.

# /opt/timelapse/edge/bootstrap.yaml

# Generated at provisioning time — DO NOT EDIT MANUALLY

device_id: TL-$(mac_address_no_colons) # e.g. TL-AABBCCDDEEFF

headend_url: https://timelapse.example.com/api

bootstrap_token: <one-time token from headend provisioning>

### 3.3.3  Local Connection Settings  /opt/timelapse/edge/local_network.yaml

Defines network connectivity. The headend may push an updated version via the configuration, which overwrites this file.

# /opt/timelapse/edge/local_network.yaml

connectivity:

preferred_order: [ethernet, wifi, modem_5g]

ethernet:

enabled: true

dhcp: true

# static:

# ip: 192.168.1.100

# gateway: 192.168.1.1

# dns: [8.8.8.8, 1.1.1.1]

wifi:

enabled: true

ssid: MyNetwork

psk: CHANGE_ME

modem_5g:

enabled: false

apn: internet

device: /dev/ttyUSB0

### 3.3.4  Device Configuration (from headend)  /opt/timelapse/edge/config.yaml

Downloaded from headend on first run and periodically re-fetched. The edge agent falls back to this cached copy if the headend is unreachable.

# /opt/timelapse/edge/config.yaml — managed by headend

device:

device_id: TL-AABBCCDDEEFF

location_name: Site Alpha - Camera 1

tenant_id: acme

schedule:

timezone: Europe/Copenhagen

capture_mode: interval # interval ¦ sunrise_sunset ¦ manual

interval_minutes: 15

active_hours: ["06:00", "21:00"] # only capture between these times

camera:

relay_gpio_pin: 18

relay_on_seconds_before: 10 # warm-up time after power-on

relay_off_seconds_after: 5 # settle time before power-off

gphoto2_port: usb: # auto-detect

initial_commands: # sent to camera after power-on

- /main/capturesettings/shutterspeed=1/250

- /main/capturesettings/aperture=8

- /main/capturesettings/iso=200

- /main/capturesettings/focusmode=mf

download_delete_after: true

quality:

check_enabled: true

blur_threshold: 80 # Laplacian variance; below = blurry

dark_threshold: 25 # Mean brightness; below = too dark

bright_threshold: 230 # Mean brightness; above = overexposed

storage:

local_path: /data/captures

circular_buffer_gb: 50

filename_pattern: "{device_id}_{timestamp:%Y%m%d_%H%M%S}.jpg"

sftp:

host: timelapse.example.com

port: 22

username: sftp_acme_site01

key_file: /opt/timelapse/edge/ssh/sftp_id_rsa

remote_base: /incoming

secondary_sftp: # optional second destination

enabled: false

host:

port: 22

username:

key_file:

remote_base:

diagnostics:

heartbeat_interval_seconds: 300

collect:

- cpu_temperature

- cpu_load

- disk_usage

- memory_usage

- battery_voltage # requires ADC or I2C sensor

- solar_voltage # optional

- connectivity_type # ethernet/wifi/5g

reverse_ssh:

enabled: false # set true by headend when remote access needed

headend_host: timelapse.example.com

headend_ssh_port: 22

headend_ssh_user: revtunnel

local_port: 22

remote_port: 0 # 0 = dynamic allocation

api:

headend_url: https://timelapse.example.com/api

token_file: /opt/timelapse/edge/api_token.txt

## 3.4  Edge Agent Operation

### 3.4.1  Capture Workflow

The edge agent follows this sequence for every capture:

- Check schedule — is it within the configured active_hours window?

- Power on camera: assert GPIO relay, wait relay_on_seconds_before.

- Connect to camera via gphoto2. Retry up to 3 times on failure.

- Send initial_commands to set camera parameters (shutter, aperture, ISO, MF mode).

- Trigger capture: gphoto2 --trigger-capture --wait-event-and-download.

- Wait for camera processing. Download image to a temp directory.

- Delete image from camera memory.

- Power off camera: de-assert GPIO relay after relay_off_seconds_after.

- Assess image quality (blur, brightness checks).

- Write metadata to local SQLite database (hash, quality score, camera state, diagnostics).

- Move image to circular buffer at /data/captures/. Enforce 50 GB limit (delete oldest if exceeded).

- Upload image to headend via SFTP (and secondary SFTP if configured).

- POST capture metadata to headend API.

### 3.4.2  Image Quality Assessment

Images are assessed for three failure conditions. A failed quality check is logged but the image is still stored and transmitted — it is flagged in the database so operators can review.

| Check | Method |
| --- | --- |
| Blur detection | Laplacian variance of greyscale image. Score below blur_threshold indicates focus loss, dirt on glass, or condensation. |
| Under-exposure | Mean pixel brightness below dark_threshold. Indicates night, lens cap, or power failure. |
| Over-exposure | Mean pixel brightness above bright_threshold. Indicates sun directly in lens or camera settings issue. |

### 3.4.3  Circular Buffer Management

# Buffer logic (runs after every capture):

# 1. Calculate total disk usage of /data/captures/

# 2. If usage > circular_buffer_gb:

# a. List files sorted by mtime (oldest first)

# b. Delete oldest files until usage is below (circular_buffer_gb * 0.9)

# 3. Log every deletion to the SQLite database

### 3.4.4  Connectivity Failover

# Network priority (handled by NetworkManager)

# 1. Ethernet (metric 100)

# 2. WiFi (metric 200)

# 3. 5G USB modem (metric 300)

# Edge agent connectivity check before SFTP upload:

# ping -c1 -W3 8.8.8.8 ¦¦ ping -c1 -W3 headend_host

# If unreachable: queue upload, retry on next heartbeat cycle

## 3.5  Systemd Services

### 3.5.1  Main Agent  /etc/systemd/system/timelapse-edge.service

[Unit]

Description=TimeLapse Edge Agent

After=network-online.target

Wants=network-online.target

[Service]

User=timelapse

WorkingDirectory=/opt/timelapse/edge

ExecStart=/opt/timelapse/venv/bin/python agent.py

Restart=always

RestartSec=10

WatchdogSec=120

NotifyAccess=main

StandardOutput=journal

StandardError=journal

[Install]

WantedBy=multi-user.target

### 3.5.2  Watchdog  /etc/systemd/system/timelapse-watchdog.service

A secondary watchdog service checks that the main agent is running and the camera relay is in a safe state. It is intentionally minimal to maximise reliability.

[Unit]

Description=TimeLapse Watchdog

After=timelapse-edge.service

[Service]

User=root

ExecStart=/opt/timelapse/edge/watchdog.sh

Restart=always

RestartSec=30

[Install]

WantedBy=multi-user.target

sudo systemctl daemon-reload

sudo systemctl enable --now timelapse-edge timelapse-watchdog

## 3.6  SSH Keys Setup

### 3.6.1  SFTP Key (Edge → Headend)

# On the edge node

sudo -u timelapse ssh-keygen -t ed25519 -f /opt/timelapse/edge/ssh/sftp_id_rsa -N ""

# Copy public key to headend provisioning

cat /opt/timelapse/edge/ssh/sftp_id_rsa.pub

# → Paste into headend create_sftp_user.sh or web admin panel

### 3.6.2  Reverse SSH Key (Edge → Headend tunnel user)

# On the edge node

sudo -u timelapse ssh-keygen -t ed25519 -f /opt/timelapse/edge/ssh/revtunnel_id_rsa -N ""

# On headend — add to revtunnel user's authorized_keys with restrictions:

# command="",no-pty,no-X11-forwarding,permitopen="localhost:*" ssh-ed25519 AAAA...

## 3.7  gPhoto2 — Camera Configuration

### 3.7.1  Verify Camera Detection

gphoto2 --auto-detect

# Expected output:

# Model Port

# Canon EOS 1300D usb:002,004

### 3.7.2  Useful gPhoto2 Commands

| Task | Command |
| --- | --- |
| List all settings | gphoto2 --list-config |
| Get a setting | gphoto2 --get-config /main/capturesettings/shutterspeed |
| Set a setting | gphoto2 --set-config /main/capturesettings/shutterspeed=1/250 |
| Capture & download | gphoto2 --trigger-capture --wait-event-and-download=5000ms |
| List camera files | gphoto2 --list-files |
| Delete all camera files | gphoto2 --delete-all-files |
| Camera summary | gphoto2 --summary |

Canon EOS 1300D — Recommended Manual Settings Focus mode: MF (manual focus — essential, camera will not wait for AF) Drive mode: Single Image format: JPG Large/Fine (RAW requires more processing time) Image stabilisation: OFF (not needed on a fixed mount) Auto power-off: OFF (camera is relay-controlled) LCD auto-off: Disable (reduces power cycling issues)

# 4.  Bootstrap & Provisioning Process

## 4.1  Overview

The bootstrap process provisions a new edge node with a unique identity and configures it to communicate securely with the headend. It runs once on first boot, then transitions to normal operation.

## 4.2  Provisioning Steps

- Operator creates a new device record in the headend web admin panel, entering the MAC address and assigning it to a tenant and location.

- Headend generates a one-time bootstrap_token and a device_id derived from the MAC address.

- Operator writes the bootstrap.yaml to the edge node (USB or pre-flashed SD card).

- Edge node boots and the timelapse-edge service starts.

- Agent reads bootstrap.yaml and calls POST /api/bootstrap with device_id and bootstrap_token.

- Headend validates the token, creates a JWT API token, and returns the config_url.

- Agent stores the JWT in api_token.txt and fetches config from config_url.

- Agent writes config.yaml and local_network.yaml. Bootstrap is complete.

- Agent POSTs a heartbeat to confirm successful bootstrap.

## 4.3  Device Identity Derivation

# device_id = 'TL-' + MAC address without colons, uppercase

# Example: MAC aa:bb:cc:dd:ee:ff → device_id = TL-AABBCCDDEEFF

import uuid

import re

def get_device_id() -> str:

mac = ':'.join(['{:02X}'.format((uuid.getnode() >> 8*i) & 0xff)

for i in reversed(range(6))])

return 'TL-' + re.sub(':', '', mac)

## 4.4  Reverse SSH Tunnel

When the headend needs to initiate a connection to an edge node (e.g. for remote diagnostics or configuration), the edge node opens a reverse SSH tunnel on request.

# Edge initiates the tunnel (triggered by config flag reverse_ssh.enabled: true)

ssh -N -R 0:localhost:22 \

-i /opt/timelapse/edge/ssh/revtunnel_id_rsa \

-o StrictHostKeyChecking=yes \

-o ServerAliveInterval=30 \

-o ExitOnForwardFailure=yes \

revtunnel@timelapse.example.com

# SSH server returns the allocated port (e.g. 40123)

# Edge POSTs to /api/reverse-ssh/ready with the port number

# Headend operator then connects:

ssh -p 40123 timelapse@localhost # on headend server

# 5.  Operations & Maintenance

## 5.1  Monitoring

| Metric | Alert Condition |
| --- | --- |
| Device online status | Last heartbeat timestamp. Alert if last_seen > heartbeat_interval * 3. |
| Image quality trend | Track blur/brightness scores over time. Alert on consecutive failures. |
| Disk usage | Alert when /data fills beyond 80%. Circular buffer should prevent this. |
| Battery voltage | Alert below threshold (configure per installation — depends on battery bank size). |
| CPU temperature | Alert above 75°C. Normal range for Orange Pi 4 Pro is 40–65°C. |
| SFTP upload lag | Alert if last uploaded image is more than 2× capture_interval old. |

## 5.2  Email Reports

The headend can send automated email reports per location, containing:

- Sample captured images (configurable: last N images, or best quality of the day).

- Quality statistics: percentage of captures passing quality checks, trend chart.

- Diagnostic summary: uptime, connectivity events, voltage/temperature charts.

- Alert summary: any quality failures, offline periods, or threshold breaches.

## 5.3  Common Issues & Remediation

| Symptom | Investigation |
| --- | --- |
| Camera not detected by gphoto2 | Check USB cable and power relay state. Run gphoto2 --auto-detect. Ensure no other process holds the USB device (kill gvfs-gphoto2-volume-monitor if running). |
| Images consistently blurry | Check camera window for condensation or dirt. Camera focus may have drifted — access required. Check blur_threshold is appropriate for the scene. |
| SFTP upload failing | Verify SSH key is still authorised on headend. Check network connectivity. Review /var/log/auth.log on headend for rejection reason. |
| Edge node offline | Check power supply voltage. Check 5G/WiFi signal. If heartbeat absent > 1 hour, consider triggering reverse SSH for diagnostics. |
| High CPU temperature | Check ventilation in housing. Verify no runaway processes (top). Reduce capture frequency if thermal throttling occurring. |
| Circular buffer not clearing | Check disk mount is healthy (df -h, dmesg). Verify timelapse user has write permission on /data/captures. |

## 5.4  Log Locations

| Component | Location / Command |
| --- | --- |
| Edge agent log | journalctl -u timelapse-edge -f |
| Edge watchdog log | journalctl -u timelapse-watchdog -f |
| Headend API log | journalctl -u timelapse-api -f |
| Nginx access/error | /var/log/nginx/access.log /var/log/nginx/error.log |
| SFTP auth log | /var/log/auth.log (headend) |
| SQLite DB (edge) | /data/timelapse_edge.db |
| PostgreSQL (headend) | sudo -u postgres psql timelapse_db |

## 5.5  Backup & Recovery

- Headend PostgreSQL: pg_dump timelapse_db daily. Store off-server (e.g. S3-compatible).

- Edge SQLite: backed up to headend during each API sync cycle.

- Captured images: stored on headend SFTP. Edge circular buffer is temporary.

- Configuration: all config versions stored in PostgreSQL config_versions table.

- SSH keys: generated per device. Public keys stored in headend DB. Rotate annually.

⚠ Before Climbing the Mast Always power-cycle the edge node remotely and confirm it boots cleanly before sending anyone up. Use reverse SSH to verify gphoto2 detects the camera before physical intervention. Bring a spare USB cable and a spare Canon battery — these are the most common failure items. Record the visit in the device notes via the headend admin panel.

# 6.  Security Checklist

Work through this checklist before deploying any production installation.

## 6.1  Headend

- Change default PostgreSQL password and restrict pg_hba.conf to localhost only.

- Set SECRET_KEY to a random 32-byte hex value (openssl rand -hex 32).

- Configure SMTP credentials — do not use a personal email account.

- Enable fail2ban: protect SSH (port 22) and the /api/bootstrap endpoint.

- Enable UFW with only ports 22, 80, 443 (and 2222 if using alternate SFTP port).

- Enable TLS with Let's Encrypt (certbot). Set up auto-renewal cron.

- Each SFTP account is chroot-jailed — verify with: sudo chroot /data/sftp/<user> /bin/true

- Review /etc/ssh/sshd_config: disable PasswordAuthentication, enable PubkeyAuthentication.

- Set up automated PostgreSQL backups and test restore procedure.

## 6.2  Edge Node

- Change default OS password immediately. Prefer key-based SSH only.

- SSH keys are per-device: never reuse keys across edge nodes.

- Bootstrap tokens are one-time use — verify they are invalidated after use.

- API JWT tokens have a configurable TTL. Set to 90 days maximum.

- Disable unnecessary services: bluetooth, avahi, cups.

- Enable automatic security updates: sudo apt install unattended-upgrades

- Do not store WiFi PSK in plaintext in version control — inject at provisioning time only.

- Verify SFTP known_hosts entry is pinned (StrictHostKeyChecking=yes in ssh_config).

## 6.3  Physical Security

- Housing should be padlocked if accessible from the ground.

- Label the housing with the device_id for maintenance identification — not with network credentials.

- Use a tamper-evident seal on the housing if the installation context requires it.

# 7.  Quick Reference

## 7.1  Service Management (Edge)

sudo systemctl status timelapse-edge # check agent status

sudo systemctl restart timelapse-edge # restart agent

sudo systemctl stop timelapse-edge # stop agent

journalctl -u timelapse-edge -f --since '1h ago' # live log

## 7.2  Service Management (Headend)

sudo systemctl status timelapse-api # check API status

sudo systemctl restart timelapse-api # restart API

sudo systemctl reload nginx # reload web config

sudo -u postgres psql timelapse_db # database console

## 7.3  Manual Capture Test (Edge)

# Run a manual capture (useful for commissioning)

source /opt/timelapse/venv/bin/activate

python /opt/timelapse/edge/agent.py --single-capture --debug

## 7.4  Configuration File Locations

| File | Path |
| --- | --- |
| Edge bootstrap | /opt/timelapse/edge/bootstrap.yaml |
| Edge network | /opt/timelapse/edge/local_network.yaml |
| Edge config (cached) | /opt/timelapse/edge/config.yaml |
| Edge API token | /opt/timelapse/edge/api_token.txt |
| Edge SSH keys | /opt/timelapse/edge/ssh/ |
| Edge SQLite DB | /data/timelapse_edge.db |
| Edge captures | /data/captures/ |
| Headend env | /opt/timelapse/headend/.env |
| Headend SFTP root | /data/sftp/ |
| Headend picture archive | /data/pictures/ |

## 7.5  Port Reference

| Port | Purpose |
| --- | --- |
| 22 (SSH/SFTP) | Headend: SSH admin access and SFTP for edge uploads |
| 80 (HTTP) | Headend: redirects to HTTPS |
| 443 (HTTPS) | Headend: API + Web UI |
| 40000–41000 | Headend: dynamically allocated reverse SSH tunnel ports |
| 8000 | Headend (loopback only): uvicorn API service |

# 8.  Power, Stability & Filesystem Design

This section documents three architectural decisions made during design review that significantly affect the reliability, power autonomy, and maintainability of deployed edge nodes: image quality analysis method, OS power management strategy, and filesystem resilience.

## 8.1  Image Quality Analysis — Method Selection

The Orange Pi 4 Pro contains a Rockchip RK3588S SoC with a dedicated NPU (Neural Processing Unit) rated at 6 TOPS. Three candidate approaches were evaluated for detecting image quality issues such as focus drift, lens contamination, condensation, and exposure problems.

### 8.1.1  Candidates Evaluated

| Method | Inference Time | Description |
| --- | --- | --- |
| OpenCV Laplacian (CPU) | < 100 ms | Computes the variance of the Laplacian of a greyscale image. Low variance = blurry. Additional checks for mean brightness detect under/over-exposure. |
| ONNX Runtime (CPU) | ~800 ms | Small no-reference quality model (e.g. BRISQUE-derived CNN) run via onnxruntime on the CPU. No special drivers required. |
| RKNN NPU | ~350 ms | Quantised neural network deployed via Rockchip RKNN Toolkit on the RK3588S NPU. Fastest inference, most power-efficient per inference, but requires proprietary SDK. |

### 8.1.2  Power Cost Comparison (per inference)

| Method | Extra Power Draw | Energy per Analysis |
| --- | --- | --- |
| OpenCV Laplacian | +0.3 W for 80 ms | 0.000007 Wh |
| ONNX CPU | +2.5 W for 800 ms | 0.00056 Wh |
| RKNN NPU | +4.5 W for 350 ms | 0.00044 Wh |

Key finding: At 1–24 captures/day, the maximum daily energy difference between any two methods is less than 0.02 Wh — under 0.02% of the total daily budget. Analysis method is irrelevant to battery sizing.

### 8.1.3  Decision: OpenCV Laplacian on CPU

Rationale (SABSA Operational Layer) Reliability: OpenCV is stdlib-level stable. RKNN requires proprietary Rockchip kernel drivers that may break on OS updates — a serious risk for unattended mast-mounted equipment. Simplicity: pip install opencv-python-headless. No SDK installation, no model conversion toolchain, no kernel module. Fitness for purpose: Laplacian variance reliably detects all primary failure modes — focus drift, dirt/condensation on glass, and exposure problems. Upgrade path: If field deployments reveal quality issues that Laplacian cannot catch, ONNX-on-CPU is the next step — one pip install, no driver risk. SABSA principle: For hard-to-reach installations, software simplicity is a security and resilience attribute.

### 8.1.4  Quality Checks Implemented

| Check | Implementation |
| --- | --- |
| Blur / focus drift | Laplacian variance of greyscale image below blur_threshold (default: 80). Detects focus drift, dirt, condensation, spider webs on glass. |
| Under-exposure | Mean pixel brightness below dark_threshold (default: 25). Detects night captures outside schedule, lens cap, or relay/power failure. |
| Over-exposure | Mean pixel brightness above bright_threshold (default: 230). Detects direct sun in lens or incorrect exposure settings. |

All three checks are configurable via the config.yaml quality section. Failed checks are logged and flagged in the database — images are still stored and transmitted so operators can review them.

## 8.2  Power Management Strategy

### 8.2.1  Energy Budget Analysis

With 1–24 captures per day, the camera is active for approximately 10–18 minutes per day. The dominant power consumer is the Orange Pi running at idle between captures.

| Component | Calculation | Energy |
| --- | --- | --- |
| Orange Pi 4 Pro — idle (24/7) | 3.5 W × 24 h | 84 Wh/day |
| 5G USB modem — idle (if fitted) | 0.5 W × 24 h | 12 Wh/day |
| Capture cycles (24×, 45 s each) | 7 W × 18 min | 2.1 Wh/day |
| Quality analysis (any method) | < 0.01 W equivalent | < 0.02 Wh/day |
| Total (no modem, always-on Pi) | — | ~87 Wh/day |

### 8.2.2  Strategy 1 — Nightly Reboot (Stability)

A daily reboot via cron at 03:00 clears memory leaks, zombie processes, and hung USB connections. gPhoto2 is known to hold USB device handles in error states that only a full reboot resolves. This is the primary stability mechanism.

# /etc/cron.d/timelapse-reboot

# Reboot every night at 03:00 local time

0 3 * * * root /sbin/shutdown -r now

# Energy cost of one reboot: ~9 W × 2 min = 0.3 Wh/day — negligible

### 8.2.3  Strategy 2 — Suspend-to-RAM Between Captures (Power)

Between capture events, the Orange Pi suspends to RAM. The RTC wakes the system shortly before the next scheduled capture. This reduces idle draw from ~3.5 W to ~0.5 W.

# Schedule wake-up and suspend (run by agent after each capture)

NEXT_CAPTURE=$(date -d 'next capture time' +%s) # calculated by agent

WAKE_TIME=$((NEXT_CAPTURE - 60)) # wake 60s before capture

# Set RTC alarm

echo 0 > /sys/class/rtc/rtc0/wakealarm

echo $WAKE_TIME > /sys/class/rtc/rtc0/wakealarm

# Suspend to RAM

echo mem > /sys/power/state

⚠ Test suspend/resume before mast deployment Resume reliability on ARM SBCs varies by kernel version and Ubuntu image. Validate at least 100 suspend/resume cycles on the bench before field deployment. The nightly reboot is the backstop — if resume ever fails, the 03:00 reboot recovers the node. Recommended rollout: deploy with nightly reboot only for 2–4 weeks, then add suspend/resume.

### 8.2.4  Combined Strategy — Energy Budget

| State | Duration / Power | Energy |
| --- | --- | --- |
| Suspended (between captures) | ~22.5 h at 0.5 W | 11.25 Wh |
| Active capture cycles (24×) | ~18 min at 7 W | 2.1 Wh |
| Nightly reboot | ~2 min at 9 W | 0.3 Wh |
| Config sync + heartbeat post-reboot | ~5 min at 7 W | 0.58 Wh |
| Total with suspend strategy | — | ~14 Wh/day |

Result: Daily consumption drops from ~87 Wh to ~14 Wh — a 6× reduction. Battery sizing for 3-day autonomy drops from ~315 Ah to ~53 Ah at 12V (standard single leisure battery).

### 8.2.5  Battery Sizing Reference

| Configuration | Daily Budget | 12V Battery (80% DoD) |
| --- | --- | --- |
| Always-on (no suspend) | ~87 Wh/day | 315 Ah for 3 days |
| With suspend-to-RAM | ~14 Wh/day | 53 Ah for 3 days |
| With suspend + 5G modem idle | ~26 Wh/day | 98 Ah for 3 days |

Battery capacity calculated assuming 80% depth of discharge (DoD) to protect battery life. Solar panel sizing should target 1.5× the daily budget to account for northern latitude winter days.

## 8.3  Filesystem Resilience — overlayFS

Power loss on a battery/solar system is a realistic failure mode — a cloudy week, a failing cell, or a connector vibrating loose on the mast. A power cut during a filesystem write can corrupt the OS, leaving the node unrecoverable without physical access. overlayFS eliminates this risk.

### 8.3.1  How overlayFS Works

The SSD root filesystem is mounted read-only. A RAM-based overlay layer sits on top — all writes during operation go to RAM. The SSD is never written to during normal operation. On reboot, the RAM overlay is discarded, and the node boots from the always-clean SSD state.

Filesystem Layout SSD (read-only): / /opt/timelapse /etc /usr /boot RAM overlay: /tmp /var/log /var/run /home /root SSD (read-write, direct mounts — bypassing overlay): /data/captures — picture circular buffer /data/timelapse_edge.db — SQLite database /opt/timelapse/edge/config.yaml — config (updated by headend) /opt/timelapse/edge/api_token.txt — API JWT token /opt/timelapse/edge/ssh/ — SSH keys

### 8.3.2  overlayFS Setup

# Install overlayroot (Ubuntu)

sudo apt install -y overlayroot

# Configure /etc/overlayroot.conf

overlayroot="tmpfs:swap=0,recurse=0"

# Add direct read-write bind mounts to /etc/fstab

# These bypass the overlay and write directly to SSD:

/dev/nvme0n1p1 /data ext4 defaults,noatime 0 2

# After enabling overlayroot, edit the underlying fstab

# (use overlayroot-chroot to access the real filesystem):

sudo overlayroot-chroot

# ... make changes ...

exit

### 8.3.3  Software Update Procedure

With a read-only root, software updates require a deliberate process that happens during the nightly reboot window:

- Headend sets update_pending flag in device config.

- Edge agent detects flag at next config sync (post-reboot heartbeat).

- Pre-reboot script (run by cron before 03:00 reboot) calls overlayroot-chroot.

- Inside chroot: apt upgrade, git pull, pip install as required.

- Exit chroot, reboot into clean updated state.

- Edge agent POSTs update_complete status to headend API.

# /opt/timelapse/edge/scripts/pre-reboot-update.sh

#!/bin/bash

# Run by cron before nightly reboot if update_pending == true

UPDATE_FLAG=/data/update_pending

[[ ! -f "$UPDATE_FLAG" ]] && exit 0

overlayroot-chroot <<'EOF'

cd /opt/timelapse/edge

git pull origin main

/opt/timelapse/venv/bin/pip install -r requirements.txt --quiet

apt-get -qq update && apt-get -qq upgrade -y

EOF

rm -f "$UPDATE_FLAG"

echo "Update complete $(date)" >> /data/update_log.txt

### 8.3.4  SQLite WAL Mode

The SQLite database is on a direct read-write mount and is therefore exposed to power-loss risk. WAL (Write-Ahead Logging) mode makes SQLite significantly more resilient to mid-write power interruption — all writes are atomic at the WAL level.

# Applied once at database initialisation (in agent.py)

import sqlite3

conn = sqlite3.connect('/data/timelapse_edge.db')

conn.execute('PRAGMA journal_mode=WAL;')

conn.execute('PRAGMA synchronous=NORMAL;') # safe with WAL

conn.execute('PRAGMA wal_autocheckpoint=100;')

conn.commit()

### 8.3.5  overlayFS Trade-offs

| Aspect | Impact |
| --- | --- |
| OS power-loss corruption | Eliminated — SSD never written during operation |
| Logs lost on reboot | Acceptable — logs sync to headend via API each heartbeat. Local logs are transient by design. |
| Config updates | Written to direct /data bind mount, survive reboot |
| RAM usage | overlayFS overlay uses RAM for writes. With nightly reboot the overlay is discarded before it grows large. Typical usage: < 50 MB. |
| Software updates | Require overlayroot-chroot procedure (automated via pre-reboot script) |
| Debugging | overlayroot-chroot provides full read-write access to the real filesystem for diagnostics |

## 8.4  Combined Architecture Summary

All Three Strategies Work Together overlayFS: SSD root is read-only → power loss cannot corrupt OS. Suspend-to-RAM: Inter-capture idle power drops from 3.5 W to 0.5 W → 6× battery improvement. Nightly reboot: 03:00 daily reboot clears USB state, memory, processes → sustained stability. Reboot also discards the RAM overlay → fresh state every morning. Config sync and heartbeat run immediately after each reboot. Software updates applied cleanly during the reboot window via pre-reboot script.

The three mechanisms address different concerns and reinforce each other. The nightly reboot is the keystone — it provides the stability backstop for suspend/resume, the clean slate for overlayFS, and the update window for software maintenance. It should always be enabled, regardless of whether the other two strategies are deployed.

# 9.  Consolidated Requirements & Decisions

This section consolidates all requirements gathered across the full design process — including decisions made during initial specification, the ChatGPT design sessions, and the Gemini SABSA architecture sessions. It serves as the authoritative requirements baseline going forward.

⚠ Purpose of This Section Requirements from earlier design sessions were spread across multiple AI tools and were partially lost or diverged over time. This section reconciles all sources into a single, versioned, agreed baseline. Every architectural decision in this document should trace back to a requirement listed here. New requirements or changes must be added here first, then reflected in the relevant technical sections.

## 9.1  Full Requirements Register

Each requirement is given a unique ID, its source, and its current documentation status. Requirements marked ⚠ were identified in earlier sessions but not yet fully addressed in this document.

### 9.1.1  Functional Requirements

| ID | Requirement | Status |
| --- | --- | --- |
| FR-01 | Timelapse capture: 1–24 images per day per camera, configurable interval. | ✓ Documented (§3.3.4) |
| FR-02 | Camera powered via GPIO relay — only energised during capture window. | ✓ Documented (§3.3.4) |
| FR-03 | gPhoto2 controls Canon EOS 1300D via USB. Camera in manual focus mode. | ✓ Documented (§3.7) |
| FR-04 | Image quality assessment after each capture (blur, brightness, obstruction). | ✓ Documented (§3.4.2, §8.1) |
| FR-05 | Images stored in local circular buffer (max 50 GB) on M.2 SSD. | ✓ Documented (§3.4.3) |
| FR-06 | Images uploaded via SFTP to headend. Each location has its own SFTP account and folder. Each camera has its own subfolder. | ✓ Documented (§2.3, §3.6.1) |
| FR-07 | Optional secondary SFTP upload destination (configurable per device). | ✓ Documented (§3.3.4 config) |
| FR-08 | Edge node creates required remote directories on SFTP server automatically. | ✓ Documented (§3.4.1) |
| FR-09 | Edge node pulls configuration from headend API at boot and periodically. | ✓ Documented (§3.3.3, §4.2) |
| FR-10 | Bootstrap via MAC-derived device_id and one-time token. Subsequent auth via JWT. | ✓ Documented (§4.2, §4.3) |
| FR-11 | Reverse SSH tunnel: edge initiates on request, notifies headend API with port. | ✓ Documented (§2.6, §4.4) |
| FR-12 | Diagnostics collected per config: CPU temp, voltage, disk, RAM, connectivity type. | ✓ Documented (§3.3.4 config) |
| FR-13 | Diagnostics and capture metadata synced to headend database via API. | ✓ Documented (§2.6) |
| FR-14 | Headend web interface: multi-tenant, shows all devices, captures, diagnostics. | ✓ Documented (§2.7) |
| FR-15 | Headend sends email reports: sample images, quality stats, diagnostics, alerts. | ✓ Documented (§2.6, §5.2) |
| FR-16 | Nightly reboot at 03:00 for software stability and USB state reset. | ✓ Documented (§8.2.2) |
| FR-17 | Suspend-to-RAM between captures to reduce idle power consumption. | ✓ Documented (§8.2.3) |
| FR-18 | Automatic timelapse video generation from captured images. | ⚠ Roadmap — not yet designed |
| FR-19 | Push notification / alert on capture failure or device offline. | ⚠ Partially — heartbeat alert documented, push channel not specified |
| FR-20 | Web interface usable by non-IT staff (simple, clear, minimal training needed). | ⚠ Requirement noted (§2.7) — UX design not yet specified |
| FR-21 | Camera image deleted from camera memory after download to edge node. | ✓ Documented (§3.4.1 step 7) |

### 9.1.2  Non-Functional Requirements

| ID | Requirement | Status |
| --- | --- | --- |
| NFR-01 | Operational temperature: -25°C to +60°C (direct sunlight on housing). | ⚠ Documented here — hardware selection guidance needed |
| NFR-02 | Power resilience: seamless failover between 230V mains and 12V battery/solar. | ✓ Documented (§3.1, §8.2) |
| NFR-03 | Filesystem resilience: power loss must not corrupt OS. overlayFS + WAL. | ✓ Documented (§8.3) |
| NFR-04 | Edge node fully autonomous when headend unreachable (local config cache, local DB, local buffer). | ✓ Documented (§1.2, §3.3.3) |
| NFR-05 | Layered / modular architecture: camera, controller, OS, central system all independently replaceable. | ✓ Documented (§1.3) |
| NFR-06 | System stability: MTBF target > 6 months unattended. Nightly reboot + watchdog. | ✓ Documented (§8.2) |
| NFR-07 | Security certifiable on roadmap: ISO 27001, CER Directive, IEC 62443. | ⚠ SABSA foundation in place — compliance gap analysis not yet done |
| NFR-08 | Cost-optimal: open source stack, no per-device licences, minimal cloud costs. | ✓ Architecture uses only open source components |
| NFR-09 | Multi-tenant: full data and config isolation between customers. | ✓ Documented (§1.3, §2.3, §2.6) |
| NFR-10 | Scalable to 100+ edge nodes without manual per-device configuration. | ✓ Pull model + SFTP provisioning script documented |
| NFR-11 | Authentication: static password in MVP, certificate + MFA in production. | ⚠ MVP strategy noted — implementation not yet designed |
| NFR-12 | No moving parts in enclosure (passive cooling only). Fanless design required. | ⚠ Documented here — enclosure spec not yet in document |

### 9.1.3  Business Requirements

| ID | Requirement | Status |
| --- | --- | --- |
| BR-01 | Business model: customer purchases hardware, supplier (us) operates the service. | ⚠ Not yet in document — pricing / SLA model needed |
| BR-02 | Remote management: all configuration changes via headend, no physical access needed for normal operations. | ✓ Pull model + reverse SSH documented |
| BR-03 | Physical access to mast/camera is costly — system must self-heal from all software faults without intervention. | ✓ Watchdog, nightly reboot, overlayFS documented |
| BR-04 | Multi-customer platform: same headend serves multiple independent customers. | ✓ Multi-tenant architecture documented |
| BR-05 | Evidence-grade data: images must be provably unmodified from capture to archive. | ✓ SHA-256 hashing at capture, logged in DB documented |
| BR-06 | Customer-facing web portal: intuitive for non-technical users. | ⚠ Requirement noted — design not yet specified |

## 9.2  Architectural Decision Log

Decisions are recorded here to preserve the rationale. Future contributors must understand why choices were made before changing them.

| ID | Decision | Rationale |
| --- | --- | --- |
| AD-01 | Orange Pi 4 Pro with RK3588S chosen as edge controller. | 8 TOPS NPU, 128GB M.2 NVMe, good Ubuntu support. Replaces earlier Orange Pi PC Plus consideration. |
| AD-02 | Ubuntu chosen over Armbian as OS. | Better long-term support, wider package availability. Armbian remains a valid alternative if Ubuntu support degrades. |
| AD-03 | gPhoto2 chosen for camera control over Motion/fswebcam. | Canon EOS 1300D is a DSLR — gPhoto2 provides full camera control (settings, RAW, EXIF). Motion/fswebcam are for webcams only. |
| AD-04 | OpenCV Laplacian chosen for image quality over RKNN NPU or ONNX. | Sufficient for all expected failure modes. No driver dependency. Full analysis in §8.1. |
| AD-05 | overlayFS chosen for filesystem resilience over full RAM boot. | Selective — OS read-only, /data mounts read-write directly. Best balance of protection and simplicity. Full analysis in §8.3. |
| AD-06 | Reverse SSH chosen over Cloudflare WARP for remote access. | No third-party dependency. Works on all networks including 5G CGNAT. Better auditability for ISO 27001 / IEC 62443 path. |
| AD-07 | Pull model chosen for configuration distribution. | Edge nodes are untrusted. Pull model eliminates inbound ports on edge. Resilient — edge falls back to cached config if headend unreachable. |
| AD-08 | SFTP with per-location OS accounts and chroot jail for image transfer. | Simple, auditable, no additional middleware. Per-location isolation. Edge can only write to its own inbox. |
| AD-09 | SQLite on edge, PostgreSQL on headend. | SQLite: no daemon, resilient with WAL mode, sufficient for single-device workload. PostgreSQL: multi-tenant, concurrent, production-grade. |
| AD-10 | Nightly reboot at 03:00 as primary stability mechanism. | Clears USB state, memory leaks, zombie processes. gPhoto2 USB hang is a known failure mode only cleared by reboot. |
| AD-11 | Suspend-to-RAM between captures for power efficiency. | Reduces idle from ~3.5W to ~0.5W. Total daily budget drops from ~87Wh to ~14Wh. Must be validated on bench before deployment. |
| AD-12 | Static password in MVP, certificate + MFA in production. | Pragmatic phased approach. MVP validates architecture. Security hardening is phase 2. |

## 9.3  Open Items — Design Work Still Required

The following requirements are confirmed but have not yet been fully designed or documented. They must be resolved before the system is considered production-ready.

| Ref | Open Item | Notes / Suggested Approach |
| --- | --- | --- |
| FR-18 | Timelapse video generation | After images are captured, the headend should be able to compose a timelapse video from a date range. Tool recommendation: FFmpeg. Triggered via API or web UI. Stored alongside source images. |
| FR-20 / BR-06 | Web UI UX design for non-IT users | The web interface must be usable by site managers and customers with no IT background. Needs: clear status indicators, thumbnail gallery, simple config editor, alert inbox. Full wireframe design needed. |
| NFR-01 | Hardware specification for -25°C to +60°C | Standard Orange Pi 4 Pro is rated 0°C to 70°C. For -25°C operation: industrial-grade M.2 SSD (pSLC), heater element or thermal mass enclosure, low-temperature grease on connectors. A specific enclosure and BOM must be defined. |
| NFR-07 | Compliance gap analysis (ISO 27001 / IEC 62443) | SABSA architecture provides the foundation. A formal gap analysis against ISO 27001 Annex A controls and IEC 62443-3-3 system requirements must be conducted before claiming compliance readiness. |
| NFR-11 | Authentication implementation (MVP and production) | MVP: bcrypt-hashed password stored in PostgreSQL, HTTPS only, rate-limited login. Production: mTLS client certificates for API, TOTP-based 2FA for web UI, per-tenant API keys. Implementation design needed for both phases. |
| BR-01 | Business model — SLA and pricing structure | Customer buys hardware (Orange Pi, camera, enclosure, power system). Supplier provides: software, headend service, monitoring, remote support. SLA targets (uptime, image delivery, support response) and pricing model to be defined outside this document. |
| NFR-12 | Enclosure specification | Fanless aluminium enclosure with passive heatsink. Must accommodate: Orange Pi 4 Pro, relay board, power supply, SSD, cable management, camera window (polycarbonate or borosilicate glass). Drip loop required on all cables. IP65 minimum. Specific model / custom fabrication decision pending. |
| FR-19 | Push notification channel for alerts | Heartbeat-based offline detection is documented. Alert delivery channel not specified. Options: email (already planned), SMS gateway (Twilio / AWS SNS), webhook to customer system. Decision needed. |

## 9.4  Remote Access Decision — Reverse SSH vs. Cloudflare WARP

During earlier design sessions (ChatGPT), Cloudflare WARP was proposed as the remote access solution. This has been reviewed and a formal decision made.

| Factor | Reverse SSH | Cloudflare WARP |
| --- | --- | --- |
| Third-party dependency | None — uses standard OpenSSH | Requires Cloudflare account, WARP client daemon on edge |
| Works on 5G / CGNAT | Yes — edge initiates outbound connection | Yes — Cloudflare tunnel handles NAT traversal |
| Auditability | Full — SSH logs, known_hosts, key fingerprints | Partial — traffic routed via Cloudflare infrastructure |
| ISO 27001 / IEC 62443 | Straightforward — standard protocol, well-documented | More complex — data transits third-party infrastructure |
| Setup complexity | Low — single SSH keypair per device | Medium — WARP client + Cloudflare tunnel configuration |
| Cost | Free | Free tier available, paid for enterprise features |
| Decision | SELECTED | Not selected for this project |

✓ Decision: Reverse SSH (AD-06) Reverse SSH is the selected remote access mechanism for all edge nodes. The edge initiates an outbound SSH connection to the headend when reverse_ssh.enabled is set in config. The headend API is notified of the allocated dynamic port via POST /api/reverse-ssh/ready. Key restriction on headend: command="",no-pty,no-X11-forwarding,permitopen="localhost:*" Cloudflare WARP may be re-evaluated if a specific customer requirement arises, but is not part of the standard architecture.

## 9.5  Hardware & Environmental Requirements

### 9.5.1  Temperature Operating Range

Requirement (NFR-01): The complete edge node must operate reliably from -25°C to +60°C. In direct summer sunlight the enclosure interior may reach higher temperatures.

| Component | Temperature Consideration |
| --- | --- |
| Orange Pi 4 Pro (SBC) | Rated 0°C to 70°C standard. For sub-zero: consider pre-heating via small resistive heater on thermostat, or thermal mass enclosure design. |
| M.2 SSD | Standard SSDs rated 0°C to 70°C. For -25°C operation: specify industrial/automotive grade pSLC SSD (e.g. Transcend MTE110S industrial, rated -40°C to 85°C). |
| Camera: Canon EOS 1300D | Rated 0°C to 40°C. At -25°C battery performance drops significantly and lubricants may stiffen. Relay-off between captures preserves battery. Consider heated enclosure for extreme cold. |
| Enclosure | IP65 minimum. Aluminium with integrated heatsink fins for passive cooling. Polycarbonate window with anti-reflective coating. Silicone gaskets rated -60°C to +200°C. |
| Connectors & cabling | Use M12 IP67 connectors for external connections. Low-temperature silicone cable jacket for outdoor runs. |
| Power supply | Specify input-side components rated to -40°C. 12V DC-DC converters: choose industrial grade with wide input range (8–36V). |

### 9.5.2  Power Architecture

The edge node must operate on either 230V AC mains or 12V battery (solar-charged) without configuration change. Failover between sources must be seamless and must not cause a reboot or filesystem write.

| Source / Function | Implementation |
| --- | --- |
| Primary: 230V AC | Via industrial DIN-rail PSU → 12V DC output. Charges battery float. |
| Secondary: 12V battery | Lead-acid or LiFePO4 battery bank. Sized per site autonomy requirement (see §8.2.5). |
| Solar input | 12V MPPT charge controller → battery. Panel sized for 1.5× daily budget. |
| Seamless failover | AC and battery connected to same 12V bus via diode OR-ing or switchover relay. Orange Pi sees only 12V — no power interruption on mains loss. |
| Low-voltage cutoff | Hardware LVC at ~11.0V (lead-acid) or ~11.5V (LiFePO4) to prevent deep discharge. GPIO signal to Orange Pi for graceful pre-shutdown logging. |
| Voltage monitoring | ADC or I2C power monitor IC (e.g. INA219) reports battery voltage to diagnostics. Configurable alert threshold in config.yaml. |

## 9.6  Business Model

The business model separates hardware ownership from service delivery. This has important implications for system architecture and support processes.

| Aspect | Detail |
| --- | --- |
| Hardware ownership | Customer purchases: Orange Pi 4 Pro, camera, enclosure, power system, mounting hardware. Supplier may assist with procurement but does not own the hardware. |
| Software & service | Supplier provides: edge agent software, headend platform, monitoring, remote configuration, updates, and support. Delivered as a managed service. |
| Access model | Supplier has remote-only access via reverse SSH (on request) and headend API. Physical access is customer's responsibility. Supplier never has unilateral physical access. |
| Data ownership | Images belong to the customer. Supplier stores them on the headend SFTP server as part of the service. Customers may additionally configure their own secondary SFTP (FR-07). |
| Update responsibility | Supplier deploys software updates via the pre-reboot update mechanism (§8.3.3). Customer is informed of major updates. Emergency patches can be deployed remotely. |
| SLA implications | System must self-heal from all software faults without physical dispatch. Physical intervention (e.g. camera refocus, enclosure cleaning) is billable on-site visit. |
| Multi-tenancy | Multiple customers on same headend platform. Full data isolation (separate SFTP accounts, separate DB tenant records, separate API tokens). No cross-tenant visibility. |
| Pricing model (TBD) | Likely: per-device monthly fee covering headend hosting, monitoring, updates, and limited remote support. On-site visits billed separately. To be defined outside this document. |

## 9.7  Security Roadmap — Towards ISO 27001 / IEC 62443

The SABSA architecture documented in this guide provides the foundation for formal security certification. The following roadmap describes the phased journey from MVP to compliance-ready.

| Phase | Focus | Controls |
| --- | --- | --- |
| Phase 1 (MVP) | Architecture & basic controls | SABSA framework documented. SHA-256 image integrity. JWT API auth. SFTP chroot isolation. overlayFS filesystem protection. SSH key auth. HTTPS TLS. Reverse SSH restricted access. Static web UI password. |
| Phase 2 (Hardening) | Production security | Certificate-based API auth (mTLS). TOTP 2FA for web UI. Per-tenant encryption keys. Signed software updates (ed25519 signatures on update bundles). Automated vulnerability scanning. Penetration test. |
| Phase 3 (Compliance) | Formal certification | ISO 27001 gap analysis and remediation. IEC 62443-3-3 assessment (industrial IoT security). CER Directive alignment (if applicable to customer sector). Formal ISMS documentation. Annual audit. |

SABSA → ISO 27001 Mapping (Summary) SABSA Contextual Layer → ISO 27001 Clause 4 (Context) and Clause 6 (Planning/Risk) SABSA Conceptual Layer → ISO 27001 Annex A.5 (Policies) and A.6 (Organisation) SABSA Logical Layer → ISO 27001 Annex A.8 (Asset management) and A.9 (Access control) SABSA Physical Layer → ISO 27001 Annex A.12 (Operations) and A.13 (Communications) SABSA Component Layer → ISO 27001 Annex A.14 (System acquisition) and A.15 (Suppliers) SABSA Operational Layer → ISO 27001 Clause 9 (Performance evaluation) and Clause 10 (Improvement)

## 9.8  Development Roadmap

| Phase | Name | Deliverables |
| --- | --- | --- |
| Phase 1 | Hardware validation | Orange Pi 4 Pro + Ubuntu + M.2 SSD + Canon EOS 1300D working end-to-end. Single capture, download, basic quality check, manual SFTP upload. overlayFS stable. Nightly reboot confirmed. |
| Phase 2 | Full edge agent | Complete Python agent: relay control, gPhoto2 capture, quality assessment, circular buffer, SFTP upload (primary + secondary), SQLite logging, heartbeat API, config pull, suspend/resume. |
| Phase 3 | Headend MVP | FastAPI backend, PostgreSQL schema, SFTP provisioning, JWT auth, basic web UI. Email reports. Bootstrap flow end-to-end with one real device. |
| Phase 4 | Multi-tenant & web UI | Full multi-tenant isolation. Customer-facing web portal. Configuration push from UI. Device management. Quality dashboards. |
| Phase 5 | Production hardening | mTLS, 2FA, signed updates, monitoring, alerting, penetration test. Industrial hardware BOM finalised. -25°C enclosure validated. |
| Phase 6 | Compliance | ISO 27001 gap analysis. IEC 62443 assessment. Video generation feature. Full documentation package for customer handover. |